7/16 DIN Male Crimp Connector For LMR400 Cable

Interface

According to                                                                                              IEC 60169-4

Electrical

Characteristic Impedance                                                                            50 ohm

Frequency Range                                                                                        DC-7.5GHz

VSWR                                                                                                      ≤1.20@DC-3000MHz

Dielectric Withstanding Voltage                                          ≥4000V RMS,50Hz,at sea level

Dielectric Resistance                                                                                    ≥10000MΩ

Contact Resistance                                                                          Center Contact ≤0.4mΩ

Outer Contact ≤1 mΩ

Mechanical

Durability                                                                                          Mating cycles    ≥500

Material and plating

Material                                    Plating

Body                                                Brass                                                            Ni

Insulator                                          PTFE                                                        －

Center conductor                            Phosphor bronze                                                  Ag

Coupling Nut                                  Brass                                                                          Ni

Crimping tube                                Annealed Copper                                                        Ni

Gasket                                            Silicone Rubber                                                          －

Environmental

Temperature Range                                                                                          -40℃～+85℃

Weatherproof Rate                                                                                                      IP 68

RoHs Compliant