7/16 DIN Female SQ Flange Mount Connector For RG402(141”))

Interface

According to                                                                                              IEC 60169-4

Electrical

Characteristic Impedance                                                                              50 ohm

Frequency Range                                                                                        DC-7.5GHz

VSWR                                                                                                ≤1.10@DC-4000MHz

Dielectric Withstanding Voltage                                          ≥4000V RMS,50Hz,at sea level

Dielectric Resistance                                                                                      ≥10000MΩ

Contact Resistance                                                                          Center Contact ≤0.4mΩ

Outer Contact    ≤1 mΩ

Intermodulation 3rd (PIM3)                                                              ≤ -165dBc@2×20W

Mechanical

Durability                                                                                            Mating cycles    ≥500

Material and plating

Material                                                              Plating

Body                                                Brass                                                                     Tri-Metal

Insulator                                          PTFE(Teflon)                                                       －

Center conductor                            Phosphor Copper                                               Ag(Silver)

Gasket                                            Silicone Rubber                                                      －

Environmental

Temperature Range                                                                                            -40℃～+85℃

Weatherproof Rate                                                                                                    IP 68

Operating Power                                                                                                        ≥500Watt

RoHs (2002/95/EC)                                                                                                  Compliant