7/16 DIN Male Right Angle Connector for 1/2” Superflexible Cable

Interface

According to                                                                                              IEC 60169-4

Electrical

Characteristic Impedance                                                                            50 ohm

Frequency Range                                                                                        DC-7.5GHz

VSWR                                                                                                  ≤1.15@DC-3000MHz

Dielectric Withstanding Voltage                                            ≥4000V RMS,50Hz,at sea level

Dielectric Resistance                                                                                ≥10000MΩ

Contact Resistance                                                                      Center Contact ≤0.4mΩ

Outer Contact    ≤1 mΩ

Mechanical

Durability                                                                                        Mating cycles ≥500

Material and plating

Material                                Plating

Body                                                Brass                                                      Ni

Insulator                                          PTFFE                                                －

Center conductor                            Phosphor bronze                                              Ag

Coupling Nut                                  Brass                                                                    Ni

Other Parts                                      Brass                                                                    Ni

Gasket                                            Silicone Rubber                                                    －

Environmental

Temperature Range                                                                                      -40℃～+85℃

Weatherproof Rate                                                                                                  IP 68