3-1/8″EIA Flange Connector For 3-1/8″Air Dielectric Coaxial Cable

Interface

According to                                                                                          3-1/8″ EIA Flange

Electrical

Characteristic Impedance                                                                            50 ohm

Operating Frequency Range                                                                          DC-3GHz

VSWR                                                                                                    ≤1.07@DC-1000MHz

Dielectric Withstanding Voltage                    ≥6000V RMS,50Hz,at sea level

Dielectric Resistance                                                                                  ≥5000MΩ

Average Power                                                                                            23KW@900MHz

Peak Power Maximum                                                                                          120KW

Contact Resistance                                                        Center Contact    ≤1.5mΩ

Outer Contact      ≤1.5mΩ

Mechanical

Durability                                                                                  Mating cycles ≥500

Material and plating

Material                          Plating

Body                                                Brass                                                Ni

Insulator                                          PTFE                                            －

Center conductor                            Phosphor bronze                                       Ag

Coupling Nut                                  Brass                                                            Ni

Other Parts                                      Brass                                                            Ni

Gasket                                            Silicone Rubber                                            －

Bolts and Washer                          Stainless Steel                                                －

Environmental

Operating Temperature Range                                                                -45℃～+85℃

Weatherproof Rate                                                                                              IP 67

RoHs Compliant