7/8″EIA Flange Right Angle Connector For 7/8″Foam Feeder Cable

Interface

According to                                                                                        7/8In EIA Flange

Electrical

Characteristic Impedance                                                                      50 ohm

Operating Frequency Range                                                                  DC-5GHz

VSWR                                                                                        ≤1.15@DC-1000MHz

Dielectric Withstanding Voltage                                    ≥6000V RMS,50Hz,at sea level

Dielectric Resistance                                                                              ≥5000MΩ

Average Power                                                                                2.3KW@900MHz

Peak Power Maximum                                                                                    90KW

Contact Resistance                                                                Center Contact    ≤1.5mΩ

Outer Contact      ≤1.5mΩ

Mechanical

Durability                                                                                    Mating cycles ≥500

Material and plating

Material                                                        Plating

Body                                                Brass                                                              Ni

Insulator                                          PTFE                                                             －

Center conductor                            Phosphor bronze                                       Ag

Coupling Nut                                  Brass                                                             Ni

Other Parts                                      Brass                                                             Ni

Gasket                                            Silicone Rubber                                            －

Bolts and Washer                            Stainless Steel                                            －

Environmental

Temperature Range                                                                                -40℃～+85℃

Weatherproof Rate                                                                                              IP 67

RoHs Compliant