Four Star Communication Parts Co.,Ltd 2000mm 1/2” Super Flexible Cable Jumper

(7/16 DIN Male to 7/16 DIN Male Connectors )

Connector Interface: CECC 22190 ;IEC 60169-4 ;DIN 47223

Electrical

Characteristic Impedance 50 ohm Frequency Range DC-3 GHz VSWR ≤1.10@698-960MHz

≤1.10@1900-2200MHz Dielectric Withstanding Voltage ≥2700V RMS,50Hz,at sea level Dielectric Resistance ≥10000MΩ Intermodulation 3rd ≤ -155dBc@2×20W

| Mechanical Durability |  |  |  | Mating cycles ≥500 |  |
| --- | --- | --- | --- | --- | --- |
| Material and Plating Body InsulatorCenter conductor Coupling nutGasket Shrinkable Tube |  | Material Brass PTFEPhosphor bronze BrassSilicone Rubber PVC |  | Plating Tri-Metal －Ag Ni －－ |  |
| Cable Family 1/2” |  |  | Superflexible cable (See below) |  |  |
| Environmental Temperature RangeWeatherproof RateRoHs 2002/95/EC | Compliant |  |  | -40℃～+85℃ IP 68 |  |

Tel:0086-511-88896930;Fax:0086-511-88892399;Email:herryma71@hotmail.com;www.fscommunication.com

Four Star Communication Parts Co.,Ltd

Construction Specification

| Material |  |  | Diameter(mm) |
| --- | --- | --- | --- |
| 1. Inner Conductor | Copper Clad Aluminum Wire or Copper | 3.55 |  |
| 2. Dielectric | Physical Foam Polyethylene | 8.70 |  |
| 3. Outer Conductor | Helical Copper Tube | 12.00 |  |
| 4. Jacket | Black PE or | 13.40 |  |
|  | Low Smoke Halogen-free Fire-retardant PE |  |  |

| Electrical Characteristics |  |  |  |  | Mechanical and Environmental Characteristics |  |
| --- | --- | --- | --- | --- | --- | --- |
| Capacitance(pF/m) |  |  |  | 80 | Min. Single Bending Radius(mm) | 50 |
| Impedance(ohm) |  |  |  | 50 | Min. Repeated Bending Radius(mm) | 125 |
| Velocity(%) |  |  |  | 83 | No. of Bends | 15 |
| Peak Power Rating(KW) |  |  |  | 19 | Mobile Apply(mm) | 350 |
| RF Peak Voltage(KV) |  |  |  | 1.13 | Bending Moment(N.m) | 5.0 |
| Insulation Resistance(MΩ.km) |  |  |  | >5000 | Tensile Strength(kg) | 110 |
| Cut-off Frequency(GHz) |  |  |  | 12.5 | Storage Temp.(℃) | -55to+85 |
| Insulation Voltage(KV) |  |  |  | 2.5 | Installation Temp. (℃) | -40to+60 |
| Inner Conductor DC Resistance(Ω/km) |  |  |  | 2.85 | Operating Temp.( ℃) | -55to+85 |
| Outer Conductor DC Resistance(Ω/km) |  |  |  | 3.50 |  |  |
| Jacket Spark(KV) |  |  |  | 5.0 |  |  |
| Shielding Effectiveness(dB) |  |  |  | >120 |  |  |
| VSWR≤(Return loss≥dB) |  |  |  |  |  |  |
| 0.005-3GHz |  | 1.15 |  | (23) |  |  |
| 0.8-1.0GHz | 1.10 |  | (26) |  |  |  |
| 1.7-2.0GHz | 1.10 |  | (26) |  |  |  |
| 2.0-2.4GHz | 1.10 |  | (26) |  |  |  |

Attenuation(VSWR1.0,cable temp.20℃) and Average Power(VSWR1.0,ambient temp.40℃)

| Frequency(MHz) | Attenuation(≯dB/100m) | Average Power(KW) |
| --- | --- | --- |
| 100 | 3.22 | 3.03 |
| 200 | 4.65 | 2.11 |
| 450 | 7.20 | 1.37 |
| 800 | 9.86 | 1.00 |
| 900 | 10.56 | 0.94 |
| 1000 | 11.15 | 0.88 |
| 1500 | 13.80 | 0.70 |
| 1800 | 15.55 | 0.63 |
| 2000 | 16.40 | 0.59 |
| 2200 | 17.35 | 0.56 |
| 2400 | 18.10 | 0.53 |
| 2500 | 18.50 | 0.52 |
| 3000 | 20.90 | 0.48 |

Tel:0086-511-88896930;Fax:0086-511-88892399;Email:herryma71@hotmail.com;www.fscommunication.com