1/4 Wavelength Type Surge Arresotor

7/16 DIN Male and 7/16 DIN Female

Dimension :

| Number | Designation | Material | Electroplating |
| --- | --- | --- | --- |
| 1 | Swivel nut | Brass rods | Tri-Metal |
| 2 | Hull | Brass rods | Tri-Metal |
| 3 | Plughole | Bronze Bar | Ag |
| 4 | Contact pin | Brass rods | Ag |
| 5 | Inner conductor | Brass rods | Ag |
| 6 | Sealing element | Silicon rubber | - |
| 7 | Dielectric | PTFE | - |
| 8 | SC | Stainless | - |

| Test item |  |  | Standard Parameter |  |
| --- | --- | --- | --- | --- |
| Withstand voltage(V) |  |  | 4000V 1min No breakdown and no arcing phenomenon |  |
| Insulation Resistance(MΩ) |  |  | ≥1x104 |  |
| Contact resistance(MΩ) | Inner conductor |  | ≤0.4 |  |
|  |  | Outer conductor | ≤0.2 |  |
| Impedance（Ω） |  |  | 50 |  |
| Frequency Range (MHz) |  |  | 700-2500 |  |
| Max. Peak Power(W) |  |  | 3000 |  |
| Discharging current(kA@8/20μS) |  |  | 30 |  |
| Residual Voltage(1kV/μs) |  |  | ＜30 |  |
| Insertion Loss（dB） |  |  | ≤0.15 |  |
| V.S.W.R. |  |  | ≤1.18 |  |
| Water-proof |  |  | IP67 |  |