Discharge Tube Type Surge Arrester SMA Male to SMA Female Port

Electrical

Characteristic Impedance                                                                                50 ohm

Dielectric Resistance                                                                                      ≥5000 M ohm

Contact Resistance                                                                            Center Contact ≤3mΩ

Outer Contact ≤2.5mΩ

AC Withstand Voltage                                      100V / 1 Min No breakdown and No Arcing Phenomenon

Frequency Range                                                                                                DC-3GHz

VSWR                                                                                                                    ≤1.15

Insertion Loss                                                                                                        ≤0.18dB

Initial Discharge Voltage                                                                    75/90/150/230/350V

Average RF Power                                                                          30W-400W(Depending on gas tube)

Peak RF Power                                                                                                      ＜500W

Maximum Discharge Current                                                                          20KA(8/20μs)

Material and Plating

Parts Name                                      Material                                                     Plating

Housing(Body)                                  Brass                                                                Ni

Insulator                                            PTFE                                                             －

Center Pin                                          Phosphor Bronze                                            Au

Inner Conductor                                Phosphor Bronze                                              Au

O-ring (Sealing)                                Silicon Rubber                                                  －

Standard Component                        Stainless Steel                                                    －

Environmental

Temperature Range                                                                                        -45℃～+85℃

Weatherproof Rate                                                                                                    IP 67

RoHs Compliant