Discharge Tube Type Surge Arrester

N Male to N Female

Interface                                                                                                According to IEC 60169-16

Electrical

Characteristic Impedance                                                                                50 ohm

Dielectric Resistance                                                                                      ≥5000 M ohm

Contact Resistance                                                                              Center Contact ≤1mΩ

Outer Contact ≤1mΩ

AC Withstand Voltage                                  2500V / 1 Min No breakdown and No Arcing Phenomenon

Frequency Range                                                                                                  DC-6GHz

VSWR                                                                                                          ≤1.15@DC-6GHz

Insertion Loss                                                                                                      ≤0.15dB

Initial Discharge Voltage                                                                      75/90/150/230/350V

Average RF Power                                                                          30W-400W(Depending on gas tube)

Peak RF Power                                                                                                    ＜600W

Maximum Discharge Current                                                                        20KA(8/20μs)

Material and Plating

Parts Name                                    Material                                                       Plating

Housing(Body)                                  Brass                                                                 Ni

Insulator                                             PTFE                                                                  －

Center Pin                                          Phosphor Bronze                                            Au

Inner Conductor                                Phosphor Bronze                                              Au

O-ring (Sealing)                                Silicon Rubber                                                  －

Standard Component                        Stainless Steel                                                  －

Environmental

Temperature Range                                                                                          -45℃～+85℃

Weatherproof Rate                                                                                                    IP 67

RoHs Compliant