| 项目名称 Item Name | 指 标 Specification |
| --- | --- |
| 频率范围 Frequency range (MHz) | 800~960/1710~2700 |
| 驻波 VSWR | ≤1.4 |
| 输入阻抗 Input impendence (Ω) | 50Ω |
| 增益 Gain (dBi) | 2±0.5/5±1 |
| 极化形式 Polarization type | 垂直极化Vertical polarization |
| 互调 Intermodulation (dBm)(2w) | ≤-90 |
| 水平波瓣宽度 Horizontal Beamwidth (°) | 360 |
| 最大功率 Max power (W) | 50 |
| 雷电防护 Lightning protection | 直流接地DC ground |
| 输入接口 Connector type | N型阴头N female |
| 重量 Weight (g) | 230 |
| 尺寸 Dimension (mm) | Φ186×85 |
| 天线罩材料 Radome material | ### 防紫外线 Uv-Protected ABS |
| 天线颜色Antenna’s color | ### 白色White |
| 环境温度 Environment Temperature (℃) | 极限温度Limit Temperature: -55~+70 工作温度Working Temperature: -40~+60 |

说明书SPECIFICATION

1、安装说明：在天花板上钻一个直径为35mm的孔，按安装图所示，用紧固螺母将天线固定在天花板上，要求天花板厚度不超过20mm。

2、贮藏运输： A、存贮在常温，干燥的环境中；

B、搬运过程中应轻拿轻放，防雨防潮。

3、注意：天线切勿接近火源等。

1、Installation instruction:  In accordance with the installation sketch,

drill a hole of Φ35mm in the ceiling.  Screw the fixing nut to fasten

the antenna on the ceiling.  The ceiling’s thickness is at most 20mm.

2、Storage and transportation: A、The antenna must be stored in room temperature and dry environment;                                      B、Handle the antenna with care.

3、Attention: The antenna mustn’t be near to fire.