SMA Female Straight Connector Edge Mount Without Nut

| Interface |  |  |  |
| --- | --- | --- | --- |
| According to |  |  | IEC 60169-15 |
| Electrical |  |  |  |
| Characteristic Impedance |  |  | 50 ohm |
| Typical Frequency Range |  |  | DC-6GHz |
| VSWR |  |  | ≤1.10@DC-3GHz |
| Dielectric Withstanding Voltage |  |  | ≥1000V RMS,50Hz,at sea level |
| Dielectric Resistance |  |  | ≥5000MΩ |
| Contact Resistance |  |  | Center Contact ≤3mΩ |
|  |  |  | Outer Contact ≤2.5mΩ |
| Mechanical |  |  |  |
| Durability |  |  | Mating Cycles ≥500 |
| Material and Plating |  |  |  |
|  |  | Material | Plating |
| Body |  | Brass | Au(Gold) |
| Insulator |  | PTFE(Teflon) | － |
| Center conductor |  | Phosphor Copper | Au(Gold) |
| Environmental |  |  |  |
| Operating Temperature Range |  |  | -40℃～+85℃ |
| RoHs | Compliant |  |  |

Tel:0086-511-88896930;Fax:0086-511-88892399;Email:herryma71@hotmail.com;www.fscommunication.com