1.0/2.3 Female Straight Bulkhead Connector For P.C.B

| Interface |  |  |  |
| --- | --- | --- | --- |
| According to | IEC 61169-29;CECC 22230;DIN47297 |  |  |
| Electrical |  |  |  |
| Characteristic Impedance | 50 ohm |  |  |
| Frequency Range | DC-4GHz |  |  |
| VSWR | ≤1.25@DC-2GHz |  |  |
| Dielectric Withstanding Voltage | ≥750 V R.M.S,At sea level |  |  |
| Dielectric Resistance | ≥1000MΩ |  |  |
| Contact Resistance | Center Contact ≤4mΩ |  |  |
|  | Outer Contact ≤2.5mΩ |  |  |
| Mechanical |  |  |  |
| Durability | Mating cycles ≥500 |  |  |
| Material and plating |  |  |  |
|  | Material | Plating |  |
| Body |  |  |  |
| Insulator | PTFE |  | － |
| Center conductor | Phosphor bronze |  | Au |
| Resilient Contact | Phosphor bronze |  | Au |
| Coupling Nut | Brass |  | Ni |
| Environmental |  |  |  |
| Temperature Range | -40℃～+85℃ |  |  |
| RoHs Compliant |  |  |  |