FME Female Right Angle Crimp Connector For RG316 Cable

Electrical

Characteristic Impedance                                          50 ohm

Typical Frequency Range                                         DC-3GHz

VSWR                                                    ≤1.25@DC-3GHz

Dielectric Withstanding Voltage                         ≥1000V RMS,50Hz,at sea level

Working Voltage                                        500V RMS,50Hz,at sea level

Dielectric Resistance                                             ≥5000MΩ

Contact Resistance                                          Center Contact ≤10mΩ

Outer Contact ≤5mΩ

Mechanical

Durability                                                Mating Cycles ≥500

Material and Plating

Material             Plating

Body                         Brass                        Ni

Insulator                      Delrin                         －

Center conductor               Phosphor Copper                     Au

Ferrule                       Annealed Copper                     Ni

Nut                          Brass                         Ni

Environmental

Operating Temperature Range                                 -40℃～+85℃

RoHs   Compliant