1.6/5.6 Male Right Angle Clamp Connector For Flex-3Cable

Interface

According to  IEC 60169-13;CECC 22240;DIN47259

Electrical

Characteristic Impedance                                          75 ohm Frequency Range                                               DC-1GHz VSWR ≤1.05@DC-100MHz

≤1.25@100-1000MHz Dielectric Withstanding Voltage ≥1000 V R.M.S,At sea level Dielectric Resistance ≥10000MΩ Peak Working Voltage  330V Contact Resistance                                       Center Contact ≤4mΩ

Outer Contact ≤2mΩ

Mechanical

Withdrawal force inner female contact                              0.5N Minimum Withdrawal force inner male contact  1.7N Minimum Insertion force between female and male                             12 N Maximum Withdrawal force between female and male                           22N Minimum Durability                                                  Mating cycles ≥500

Material and plating

Material Plating Body Brass                 Au/Ni Insulator                      PTFE－Center conductor  Phosphor bronze  Au Resilient Contact  Phosphor bronze  Au Coupling Nut                  Brass                  Ni Other Parts                    Brass                  Ag

Environmental

Temperature Range                                           -40℃～+85℃ RoHs Compliant