T43 HDC (SMZ) U-Link Without Test Port

(9mm Centre Spacing)

Interface

According to                                               BS9210F0022

Electrical

Characteristic Impedance                                         75 ohm

Frequency Range                                               DC-3GHz

VSWR                                                   ≤1.30@DC-1GHz

Dielectric Withstanding Voltage                    ≥1500V RMS,50Hz,at sea level

Dielectric Resistance                                           ≥5000MΩ

Contact Resistance                                       Center Contact ≤5mΩ

Outer Contact ≤1mΩ

Mechanical

Durability                                             Mating cycles ≥250

Material and plating

Material               Plating

Body                         AL                        Purple Plating

Insulator                      PTFE                      －

Center conductor               Beryllium bronze                     Au

Spring Contact                 Phosphor bronze                     Au

Environmental

Temperature Range                                           -40℃～+85℃

RoHs Compliant